On average - Hard disk stocks remain empty for 12.8 hours on depletion

Empty 20 times - 71% of simulation time

On average - Aluminium stocks remain empty for 10.7 hours on depletion

Empty 15 times - 44.5% of simluation time

** Single Run - get 95% CI from trial of 125 runs

Removing the constraint for testing purposes, we see that if there were never capacity issues in the system,

- The plate manufacturing activity would see 120% engagement on average (across the two machines)

- The hard disk assembly activity would see 43% engagement on average (across the two machines)

Aluminium

Currently system has a capacity of 50.

Used for fixed duration of 8 minutes.

=> 120% engagement over 15 hours across the 2 Roll and Cut machines. This would be 1080 minutes active, which would constitute 135 items processed

=> Set reorder quota to 136 (reorders once it drops below 136)

=> Setting Q = ?, s = 136

How do we justify Q? Could do Q = 115; Reasoning, taking that the company don't want to have to restock more than once per day (No reason - idk?!)

To have enough stock to see them through the 15 hours of restock process plus extra 9 hours until restock need expected number of items in that duration;

24*60*120%/8 = 216 (round to nearest 10 so 220)

* Set total capacity to 220+something. Maybe base on the lower 95% CI for engagement (eg. something = 220-Expected remaining stock at lower 95% CI engagment bound)

=> In the default state the stocks were always empty by the time the restock occured - in this updated system that may not always be the case,

so do not completely fill capacity to allow for this (Using 95% bound => in current system expect no overcapacity 95% time)

Currently system has a capacity of 50.

Used for duration distributed by Beta(3,2)+1; => Expected duration = 3/(3+2) + 1 = 1.6 minutes

=> 43% engagement over 15 across the 2 workbenches would be 387 minutes active, which would constitute 241.875 items processed on average

=> Set reorder quota to 242

=> Setting Q = 390 (rounded up from 387), s = 242

* Same justification as above - same comment re capacity should be slightly higher.

DID OTHER GROUP CHANGE STARTING RAW MATERIAL STOCKS TO THEIR NEW Qs? (Doesn't really matter as think will be fixed by warmup)

While these results prove very strong we recognise that these updates require a major expansion of the existing raw material capacity(almost 350% and 470% increases), to a point which be beyond practical feasiblilty at this time.

As such, while a major capacity expansion is vital, we have also provided projections for the updated metrics of more conservative raw material stock control overhauls;

Which to test, 2/3?

150% and 200% capacity increases

250% and 350% capacity increases